OWL Building Control Solutions –

Building Control & Building Regulations –

General Guidance

The Building Regulations (2010) set standards that aim to protect the safety, health and welfare of people in and about buildings.

These regulations apply to all types of building work, whether it’s constructing a new property, changing the use of an existing one, or extending/altering current structures.

Recent updates to the Building Regulations have

introduced several significant changes that all designers & builders should be aware of.

From June 2022, requirements for new dwelling-houses include Part S (infrastructure for charging electrical vehicles) and Part O (overheating). Furthermore, requirements relating to Part F (ventilation) and Part L (conservation of fuel and power) have become more stringent.

Looking ahead, designers and builders should anticipate further changes to requirements for Part L under the Future Homes Standard, expected to come into effect in 2025.

Changes to Building Regulations often arise in response to societal issues. Climate change was the key driver for the introduction of Part O, with the primary intention of addressing the increasing health risks associated with overheating.

Meanwhile, the Building Safety Act (2022), which enabled the introduction of the Duty-Holder Regime, was a response to the Grenfell Tower disaster in 2017.

For designers, builders and renovators, these regulations have practical implications for planning, designing and building, affecting costs and timelines. Engaging with experienced professional consultants, such as architects, energy assessors and structural engineers (and more), early on in your project, can help navigate these regulations effectively. It’s also beneficial to approach your chosen building control body (BCB), whether a registered building control approver or local authority, from the design and planning stages, so that necessary adaptions to any plans are more straightforward.

The Building Regulations are minimum standards. As such, any designer or builder and client must ensure adherence to their requirements. A building control application with a registered building control approver (initial notice) or local authority (full plans or building notice) must be made before commencing any construction works. To assist any BCB in reviewing your application, it’s vital to have a comprehensive construction specification and technical design in place.

To stay compliant, you should maintain clear communication with your BCB throughout your project, including requests for site inspections, submission of amended designs and commissioning certification at the end of the project. The BCB also requires notifications at various stages, including a compliance declaration at the end of the project.

Staying informed about changes to Building Regulations involves continuous learning and engagement. Accessing continuing professional development (CPD) resources and consulting with experienced professionals can enhance your understanding and help you to gain confidence your project partners are to responding to evolving requirements.

CHANGES TO THE BUILDING REGULATIONS 2010 (as amended)

INFORMATION FOR DUTYHOLDERS

Background and what has changed

The Government has published amendments to the Building Regulations, which will apply to all applications for building control consent from 1 October 2023.

In addition, the Government has formed a new regulatory body within the Health and Safety Executive (HSE) called the Building Safety Regulator (BSR). The BSR will oversee many aspects of building safety, including Building Regulations and Building Control.

If you, or someone on your behalf, have submitted a building control application, you have new legal responsibilities from 1 October 2023 as a Client, Designer or Contractor. You must follow these legal procedures so that your building control application can be considered.

If you do not follow the procedures prescribed for your role, it will result in your application being rejected or reverted to local council control, which will require the same information.

Registered Building Control Approvers

Approved inspectors are independently monitored and regulated by the Construction Industry Council Approved Inspector Register (CICAIR) under the Building Act 1984. Under the new building control system all Approved Inspectors will need to become Registered Building Control Approvers (RBCA) which requires registration with the BSR.

The registration process will begin in October 2023 with all Approved Inspectors expecting to have applied for registration before 6 April 2024 when this becomes a requirement.

Registered Building Inspectors

Building control is to become a regulated profession, meaning that the job title ‘Building Inspector’ will be protected in law. There is a requirement for certain qualifications or experience to be held by individuals working within the profession and for them to register with the regulator (BSR) as Registered Building Inspectors (RBIs). This will become a requirement from April 2024 and applies to individuals within Local Authorities and those working for Approved Inspectors.

All RBIs will have their competence third party validated against the Building Inspector Competency Framework for the class of registration they are applying for. This will help ensure high performance and consistency across all sectors.

What is your dutyholder role?

The following table identifies who you are prescribed in the new Part 2A Dutyholders and Competence. The diagrams that follow the table summarise your new legal duties and roles and what information your Approved Inspector needs from you throughout the processing of your application.

| Identifying where you fit: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A person for whom the project is carried out |  |  |  | CLIENT |  |  |  |  |  | DOMESTIC CLIENT |  |  |  |  | Same as the Client, but not for the furtherance of a client's business |  |  |
| The Designer appointed under Reg 11D – is usually the lead designer. |  |  |  | PRINCIPAL DESIGNER |  |  |  |  |  | PRINCIPAL CONTRACTOR |  |  |  |  | The Builder appointed under Reg 11D – is usually the lead contractor. |  |  |
| Any person (including a client, Builder or anyone else) who carries out, arranges for or instructs design. |  |  |  | DESIGNER |  |  |  |  |  | CONTRACTOR |  |  |  |  | Any person (including a Client (but not a domestic client) who carries out, manages or controls work |  |  |

Note: All dutyholders must share information with anyone who might be affected within the Client, Designer and Contractor chain.

What are your duties?

CLIENT is any person for whom a project is carried out.

You must: - Ensure suitable arrangements exist to plan, manage and monitor the building work to ensure compliance with the Building Regulations. - Ensure that these arrangements are maintained throughout the life of the project. - Take all reasonable steps to ensure any designers or contractors appointed are competent to carry out the work for which they are appointed. - Notify the Approved Inspector when you appoint Principal Designer and Principal Contractors, even if they are sole designers or contractors. Including: - Name, address, telephone number and email of these parties. - Details of any change in the dutyholders and the dates of their appointment. - A signed statement of authority that the information is, to the best of your knowledge, correct. - Let the Approved Inspector know when the building work is commenced. See notes below on what is deemed commencement. - When the work is complete, you must provide notification to the Approved Inspector, which contains the following information: - The Client's name, address, telephone number and email address. - The Principal Designer's and Principal Contractor's names, addresses, telephone numbers and email addresses. - A statement that the building work is complete. - A signed statement that, to the best of your knowledge, the building work complies with the building regulations. - A signed statement from each Principal Designer and Principal Contractor that they have fulfilled their duties under the building regulations. The Approved Inspector cannot issue its final certificate without receiving these signed declarations.

DOMESTIC CLIENT is a client for whom a project is carried out which is not in the course or furtherance of the Client's business (whether for profit or not).

You must: - Ensure suitable arrangements exist to plan, manage and monitor the building work to ensure compliance with the Building Regulations. - Ensure that these arrangements are maintained throughout the life of the project. - Take all reasonable steps to ensure any designers or contractors appointed are competent to carry out the work for which they are appointed. However: - The Principal Contractor or contractor must carry out these duties. Unless: - The Client agrees that the Principal Designer can carry out these duties.

PRINCIPAL DESIGNER is usually the lead designer and is appointed under Regulation 11D (principal designer and principal contractor) to perform the duties of a Principal Designer.

You must: - Perform all the functions listed within the Designer roll below, and, In addition, you must: - Plan, Manage and Monitor the design work during the design phase. - Coordinate all matters relating to design work to ensure that it will comply with the Building Regulations if building work is carried out. - Ensure dutyholder cooperation. - Ensure all designers coordinate their designs so that building work will be carried out following the designs and will comply with the Building Regulations. - Ensure all designers comply with their duties. - Liaise with the Principal Contractor. - Have regard to comments from the Principal Contractor about compliance with the Building Regulations. - Assist the Client in providing information to designers. - Review the arrangements of any previous Principal Designer. - Notify Building Control in writing, where applicable, that the work is being carried out on behalf of a Domestic Client. - Where applicable, sign the declarations as referred to above in the Client's duties when work is complete.

DESIGNER means anyone (including the Client, contractor or anyone else) who, during their business, carries out design work or arranges for or instructs anyone under their control to do so.

You must: - Plan, manage and monitor design work so that if the building work were carried out, it would comply with the Building Regulations. - Cooperate with the Client, designers, and contractors to the extent that if the building work were carried out, it would comply with the Building Regulations. - Not start any design work unless you are satisfied that the Client is aware of the duties owed by the Client. - Carry out design to ensure that if building work were carried out, the design it would comply with the Building Regulations. - Provide sufficient information about the building's design, construction and maintenance to allow the Client, other designers and contractors to comply with the Building Regulations. - Consider any other design work and report any compliance concerns relating to the design compliance to the Principal Designers and Client. - Must advise the Principal Designer or the Client whether any work they are designing is Higher-Risk building work.

PRINCIPAL CONTRACTOR is usually the main contractor and is appointed under Regulation 11D (principal designer and principal contractor) to perform the duties of a Principal Contractor where there is more than one contractor.

You must: - Plan, Manage and Monitor the building work during the construction phase. - Coordinate matters relating to building work to ensure the building work complies with the Building Regulations. - Ensure cooperation amongst all dutyholders. - Ensure all building work is coordinated so that it complies with the Building Regulations. - Ensure contractors comply with their duties. - Liaise with the Principal Designer as required. - Have regard to comments from the Principal Designer concerning compliance with the Building Regulations. - Assist the Client in providing information to contractors. - Review the arrangements of any previous Principal Contractor. - Notify Building Control in writing, where applicable, that the work is being carried out on behalf of a Domestic Client. - Sign the declarations as referred to above in the Client's duties when work is complete.

CONTRACTOR is any person (including the Client, but not a domestic client) who, in the course of a business, carries out, manages or controls any building work.

You must: - Plan, manage and monitor the design work such that if building work were carried out following it, it would comply with the Building Regulations. - Cooperate with the Client, designers and contractors to the extent that if building work were carried out following the design, it would comply with the Building Regulations. - Not start any design work unless they are satisfied that the Client is aware of the duties owed by their duties. - Ensure the building work they are carrying out complies with the Building Regulations. - Provide each worker under their control with appropriate supervision, information and instruction to ensure the building work complies with the Building Regulations. - Provide sufficient information about the work to allow the Client, other designers and contractors to comply with the Building Regulations. - Consider other building works when you are only carrying out part of the building work and report any concerns relating to compliance to the Principal Contractor. - Provide advice to the Principal Contractor or the Client on whether any work is Higher-Risk building work.

Notification of Dutyholders

The Client must give notification to the Approved Inspector when they appoint a Principal Contractor (or Sole Contractor) or a Principal Designer (or Sole Designer)

The notice must be given to the Approved Inspector and include,

- The name, address, telephone number and email address of the Dutyholder (PC or PD or both).

- If the appointment is a change of Dutyholder (PC or PD or both), the name, address, telephone number and email address of the previous Dutyholder (PC or PD or both) and the date their appointment ended.

- If the notice is given on behalf of the client, a signed statement from the client that they agree to the notice being given and that the information in the notice is correct.

Where the client is a Domestic Client, the relevant notices must be given to the Approved Inspector by the PC or PD and include a statement that the notice is given on behalf of a Domestic Client

What is deemed the commencement of work?

Notice before starting work and notice when work is deemed "commenced."

To serve the Initial Notice, the client must advise the Approved Inspector of the date when it is proposed that sufficient work will have been carried out to deem the work as "commenced".

Where work does not involve foundation or substructure works, the client must state what they consider will amount to 15% of the work described in the Initial Notice.

Work being deemed as commenced is,

- For the construction of a complex building,

  - work is to be regarded as commenced concerning that building or the first stage of building work when the foundations supporting the building and the structure of the lowest floor level of that building (but not the other buildings or structures to be supported by those foundations) are completed.

- Where the work consists of (a) the construction of a building that is not complex or (b) the horizontal extension of a building,

  - work is to be regarded as commenced when the sub-surface structure of the building or the extension, including all foundations, basement levels (if any) and the structure of the ground floor level, is completed.

- Where the work consists of any other building work, then work is to be regarded as commenced when the initial work is completed ("initial work" means the work set out in the initial notice, which the client considers amounts to 15% of all the work described in the initial notice)

After three years, the Initial Notice will automatically cease to have effect unless sufficient work (as above) has been carried out to deem the project commenced AND the client has given written notice to the Approved Inspector.

final certificate requirements

Compliance Declarations

Where work described in an Initial Notice is complete, the Client must give to the Approved Inspector notice to this effect. The notice must include,

- The name, address, telephone number and email address of the Client.

- The name, address, telephone number and email address of the Principal Contractor and the Principal Designer.

- A statement that the building work is complete.

- A statement signed by the client that, to the best of the Client's knowledge, the building work complies with the Building Regulations.

- A statement given by each Principal Contractor for the work and each Principal Designer for the work signed by the person to which the declaration relates and includes,

  - The name, address, telephone number and email address of that person

  - The dates of their appointment

  - Confirmation that they fulfilled their duties under the Building Regulations (see details of duties above).

You should note that the Approved Inspector cannot issue its Final Certificate without receiving the signed declarations detailed in the last two (black) bullet points above, as the Final Certificate requires the Approved Inspector to declare that they have received these signed statements.

Fire Safety Information

Regulation 38 requires information relating to fire safety to be passed on at the end of a project on all buildings to which the Regulatory Reform (Fire Safety) Order 2005 applies.

Under the Amendment Regulations we are required to make a statement on our Final Certificate confirming that we have received confirmation that Regulation 38 has been complied with.

You should note that the Approved Inspector cannot issue its Final Certificate without receiving confirmation that Regulation 38 has been complied with.

Transitional Provisions

As with all Building Regulations there is some fixability on the application of the changes at the early stages of their implementation, these are called transitional provisions.

Work other than Higher-Risk Buildings

The Amendment Regulations, which include duty holder and competence requirements does not apply where:

- an Initial Notice is given and accepted by the relevant Local Authority before 1 October 2023; and,

- work commences before 6 April 2024.

For any new Initial Notices served after the 1 October 2023, the Amendment Regulations will apply.

Higher-Risk Buildings

From 1 October 2023, Building Control for alterations to existing, or the design and construction of new Higher-Risk Buildings (HRBs), can only be undertaken by the BSR. There are however some circumstances where approved inspectors may continue as the Building Control provider on HRB projects.

The BSR process will involve passing stringent ‘gateways’ at planning, design, and construction/occupation stages. The BSR has advised that they will administer this process by utilising the expertise of Multi-Disciplinary Teams (MDTs) which may include suitably qualified and experienced RBIs from the public and private sector.

HRBs are defined as buildings with 7 or more storeys or have a storey height of 18m or higher and contain either at least two residential units, or, during design and construction, are a hospital or care home. HRBs are defined in detail within the Higher-Risk Buildings (Description and Supplementary Provisions) Regulations 202310. Additional guidance on determining if a building is a HRB has been published by the Government.

There is some flexibility for projects which are currently being worked on by Approved Inspectors, these have been termed ‘in-flight’ projects.  An in-flight project is excluded from the changes to the Building Regulations and Building Control system, i.e., they may continue to use the incumbent approved inspector instead of the BSR in the following circumstances:

- An Initial Notice has been given and accepted by the relevant Local Authority before 1 October 2023.

- Work is ‘substantially progressed’ – this means the pouring of concrete for foundations, including piling. Or where work relates to an existing building, when the work has started.

- The Approved Inspector has registered to become a RBCA and employees a suitably qualified registered Building Inspector(s) before 6 April 2024.

OWL Building Control Solutions –

Building Control Process Update

Due to the changes within the wider construction industry and more specifically Building Control – the way that OWL BCS operate as a Building Control Body has changed. You may have previously received correspondence regarding the re-submission of initial notices – this has been confirmed as an erroneous statement from Department for Levelling Up Housing and Communities (HM Government).

What are the confirmed changes?

OWL BCS will become a Registered Building Control Approver from the 6th of April 2024 & become subject to The Building (Registered Building Control Approvers etc.) (England) Regulations 2024 (RBCA Regs). All those completing ‘restricted activities’ for Building Control purposes (Plan checking / Site inspections) will become Registered Building Inspectors (RBIs) from the 6th April 2024.

OWL BCS contract provides details (terms of engagement letter) which will provide guidance regarding the specifics of this arrangement (in accordance with Annex 2 of the Professional Conduct Rules for RBCAs & Annex 2 of the Code of Conduct for RBIs).

How does this impact me?

Irrespective of the date of the initial notice & whether your project has started OR commenced - all legal notices (e.g. Final certificates) produced after the 6th of April will now be in the form as outlined within Schedule 1 of the RBCA Regs 2024. Furthermore – all restricted activities (inspections & plan checks) will now be completed by Registered Building Inspectors (RBIs).

OWL BCS and individual RBIs will need to act in accordance with the professional codes and standards as published by the Building Safety Regulator (link below).

https://www.hse.gov.uk/building-safety/building-control/codes-standards.htm#

Due to these changes – the way that you interact with Registered Building Inspectors & OWL BCS (as a Registered Building Control Approver) will differ from your previous interactions with Building Control Body(s).

OWL BCS remain committed to providing an exemplary Building Control service for our existing and prospective clients.

OWL Building Control Solutions:

Guidance Note –

Domestic Clients & Appointment of Duty-Holders

- WHAT HAS CHANGED

The Government has published amendments to the Building Regulations, which applies to all applications for building control (submission of Initial Notice) from 1st October 2023.

- WHAT IS THE LEGISLATION

Chapter 2A of the Building Regulations 2010 (as amended) (BR 2010) imposes various requirements regarding duty-holders. The Building (Registered Building Control Approvers etc.) (England) Regulations 2024 (RBCA Regs 2024) imposes various requirements relating to duty-holders informing their appointed RBCA (e.g. OWL BCS) during a projects progression.

- DOMESTIC CLIENTS & APPOINTMENT OF DUTY-HOLDERS

Domestic clients have differing requirements / duties / responsibilities in comparison to ‘clients’.

- WHAT IS A DOMESTIC CLIENT vs A CLIENT (sourced from BR 2010 as amended)

Client - “client” means any person for whom a project is carried out.

Domestic Client - “domestic client” means a client for whom a project is being carried out which is not in the course or furtherance of a business of that client.

Business - “business” means a trade, business or other undertaking (whether for profit or not).

- DOMESTIC CLIENT vs CLIENT DUTIES (sourced from BR 2010 as amended)

Clients must; ‘make suitable arrangements for planning, managing and monitoring a project (including allocation of sufficient time and other resources) so as to ensure compliance with all relevant requirements’. For Domestic Clients – these responsibilities must be carried out by the principal contractor for the project (in some instances – the principal designer).

- APPOINTMENT OF DUTY-HOLDERS (sourced from BR 2010 as amended)

Clients & Domestic Clients should appoint a Principal Designer & Principal Contractor prior to the construction phase of the project (Reg 11D BR 2010). If Domestic clients do not make these appointments: (a)the designer in control of the design phase of the project is the principal designer; (b)the contractor in control of the construction phase of the project is the principal contractor.

- APPOINTMENT OF DUTY HOLDERS – INFORMING OWL BCS (sourced from RBCA Regs 2024 as amended)

A notice (of the appointment of the duty-holder) must be given to the approver (e.g. OWL BCS) within the period of 14 days beginning with the date of the appointment. (see OWL BCS Notice of Duty-Holders Document).

- WHO CAN BE A CONTRACTOR / PRINCIPAL CONTRACTOR (sourced from BR 2010 as amended)

Contractor -“contractor” means any person (including a client, but not a domestic client) who, in the course of a business, carries out, manages or controls any building work.

Principal Contractor -“principal contractor” means the contractor appointed under regulation 11D (principal designer and principal contractor) to perform the duties of a principal contractor under these Regulations;

- PRINCIPAL CONTRACTOR – COMPETENCE (sourced from BR 2010 as amended)

A principal contractor must have— the skills, knowledge, experience and behaviours necessary to fulfil the duties of a principal contractor under these Regulations in relation to the building work included in the project.

- WHO CAN BE A DESIGNER / PRINCIPAL DESIGNER (sourced from BR 2010 as amended)

Designer - “designer” means any person (including a client, contractor or other person referred to in Part 2A of these Regulations) who in the course of a business— (a) carries out any design work, or (b) arranges for, or instructs, any person under their control to do so.

Principal Designer -“principal designer” means the designer appointed under regulation 11D (principal designer and principal contractor) to perform the duties of a principal designer under these Regulations;

- PRINCIPAL DESIGNER – COMPETENCE (sourced from BR 2010 as amended)

A principal designer must have— the skills, knowledge, experience and behaviours necessary to fulfil the duties of a principal designer under these Regulations in relation to the design work included in the project.

- COMPLIANCE DECLARATIONS (sourced from RBCA Regs 2024 as amended)

Prior to issue of a final certificate – an RBCA (e.g. OWL BCS) must receive compliance declarations from all duty-holders (including Domestic Clients / Principal Contractors & Principal Designers) associated with the project.

Compliance declarations take the form of:  ‘confirmation—

(aa)in the case of a principal contractor (or sole contractor), that they fulfilled their duties as a principal contractor under Part 2A (dutyholders and competence) of the 2010 Regulations.’

(bb)in the case of a principal designer (or sole or lead designer), that they fulfilled their duties as a principal designer under Part 2A (dutyholders and competence) of the 2010 Regulations.

- CONCLUSION

Domestic Clients, Principal Designers & Principal Contractors have a range of duties and responsibilities under the Building Regulations 2010 (as amended) & RBCA Regulations 2024 (as amended) – only an overview has been provided in this document. Domestic Clients cannot act as Principal designers OR Principal contractors (section 8 & 10 above).

All duty-holders are required to confirm (in writing) that they have performed all their necessary duties in the form of a compliance declaration – prior to a final certificate being issued.

There may be forthcoming guidance / legislation which varies the legal position of those completing works on their own homes – from that of the legislation outlined above. OWL BCS await this clarification & will inform all clients accordingly.

- FURTHER GUIDANCE

Design and building work: meeting building requirements - GOV.UK (www.gov.uk)

THE BUILDING ACT 1984 (AS AMENDED)

THE BUILDING SAFETY ACT 2022 (AS AMENDED)

THE BUILDING REGULATIONS 2010 (AS AMENDED)

THE BUILDING (REGISTERED BUILDING CONTROL APPROVERS ETC.) (ENGLAND) REGULATIONS 2024

Disclaimer:

This guidance note provides isolated guidance regarding a specific aspect associated with legislation changes. The contents of this document note are not intended to replace any responsibility / requirements or liability relating to the need for all duty-holders to read, interpret, understand and comply with the relevant regulatory requirements for their proposed building works. OWL BCS produce the guidance note in good faith and accept no liability for mis-interpretation or misunderstanding resulting in non-compliance to any applicable legislation.